| Week |  | xx |  |  |  |  |  |  | xx |  |  |  |  |  |  | xx |  |  |  |  |  |  | xx |  |  |  |  |  |  | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | Day | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | min |
| Nursery tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Organic waste collection - Drive vehicle to the market - Take baskets with vegetable/fruit waste from vendors - Replace with empty baskets |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |
| 2 | Waste sorting (M1) - Empty basket on the concrete floor - Remove inorganic materials and place in separate basket - Remove carbon/cellulose/fibre rich organic materials and place on compost heap | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | W | 60 |
| 3 | Pallet harvesting (M2) - Move 1st pallet near to the sieving machine (see Appendix E for the handling schedule) - Put buckets under each of the three sieve outlets to collect separated streams - Turn-on sieving machine and conveyor - Turn-on bulk balance, tare it with empty crate - Take the first crate from the pallet, weigh it, record the weight and empty the crate in the conveyor hopper - Repeat this for all crates on one pallet - Replace full buckets under outlets when full - Take next pallet, repeat the steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 |
| 4 | Manual larvae separation from residue (M3) - Remove particles from buckets with larvae - Weigh the larvae, record weight, repeat for other buckets of one pallet - Repeat the steps for buckets from next pallets |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |
| 5 | Residue packing (M4) - Take fine residue, put into bags: add a bit of water on top of the bag if the material is hot. Put in compost storage room and leave for a month - Take big residue put onto composting heap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |
| 6 | Larvae cleaning - Prepare 8 buckets with tofu cloths - Hang tofu cloths inside the buckets - Put 5 kg of harvested larvae/bucket - Wash larvae with water until they are clean Let larvae drip out, remove from cloth and place into large container with cocopeat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |
| 7 | Waste shredding (M5) - Prepare waste transfer bucket under shredder - Shred all the separated waste on the concrete floor - Replace full transfer bucket, weigh full bucket and add to IBC tank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 |
| 8 | Prepare and set-up small box (SB) - Prepare (36) clean small boxes - Add 1 kg of fruit waste (⅘) and PKM (⅕) per SB - Place 4 SB into 1 crate (4x 30x20 cm) - Repeat the process until 36th SB are done - Weigh (36) cups of 10,000 5-DOL and add to SB - Put the crates into SB frame and add label |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |
| 9 | Set-up pallet - Take crates with in total 36 SB from oldest date - Prepare 36 conversion crates with pallet and 5 ventilation frames - Weigh 9kg of shredded waste, put into a conversion crate and add content of one SB - Repeat steps for all 36 crates - Setup pallet adding 6 crates per row and cover with frame, repeat 5 times for each pallet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |
| 10 | Composting maintenance (M6) - Add waste from collection, harvesting and nursery operation onto newest compost pile - Turn over compost pile - Turn over older compost piles once per week |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |
| 11 | Crates & buckets cleaning - Move all materials from nursery and waste treatment operation to cleaning area in compost building - Clean with high pressure washer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 |
| Upkeep tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| U1 | Clean waste shredding area - High pressure washer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| U2 | Clean harvesting area - Sweep the floor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| U3 | Clean composting area - High pressure washer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 |
| U4 | Clean compost storage - Sweep the floor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |
| U5 | Clean storage room - Sweep the floor & surfaces |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 |
| U6 | Wash tofu cloths (60-900C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 |
| Measuring/Monitoring tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M1 | Waste sorting - Measure and note down weight of waste received from different waste sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| M2 | Pallet harvesting - Measure and note down weight of each crate of one pallet by placing stacks of 3 on the scale |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| M3 | Manual residue separation - Measure and note down weight of larvae after manually removing residue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| M4 | Residue processing - Measure and note down weight of fine residue after bagging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| M5 | Waste shredding - Measure and note down weight of waste after shredding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| M6 | Compost maintenance - Measure and note down weight of different residue streams before piling them |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 640 |